EE368 Chapter 1 – Introduction – Segment: Digital Images

SHOT FRAMING: SUPERIMPOSE

SLIDE 1

Hello, and welcome!

I am Bernd Girod, a professor in the Department of Electrical Engineering at Stanford University. I am delighted to teach this course on Digital Image Processing. My own research interests are closely related to the contents of this course, so I am particular excited to introduce you to this dynamic field with broad impact.

The Stanford course number is EE368, and the course is also cross listed as a computer science course with the number CS232.

In this introductory chapter, I want to give you an idea about what digital images are, why we process them, and what kind of problems you will be able to solve with the material you will learn in this course. Finally, we’ll talk a little bit about the course organization.

SLIDE 2

We begin by the question “What is an image?”

The word “image” has many different meanings – generally a likeless of something, typically a picture. Our definition, for purposes of this course, is much more technical and narrow.

CLICK

What you see here is a woodcut by Albrecht Duerer from 1525 illustrating a renaissance imaging technique. The artist in the picture uses the tip of a rod to fix the vantage point of a picture he is drawing, along with a grid of strings to subdivide the visual field.

CLICK (SLIDE 3)

This grid forms an x,y coordinate system which is reproduced on the drawing paper.

CLICK

This leads us to the definition of an image. For the purposes of this class, an image is a visual representation in form of a function f(x,y) where f is related to the brightness (or color) at point (x,y)

CLICK

Most image are defined over a rectangle.

CLICK

And, image are naturally continuous in amplitude and space.

SLIDE 4

Let’s talk about Digital Images and Pixels.

You can process a continuous image by optical means, but only in a very limited way. Digital circuitry or a computer requires digitization.

CLICK

Digital image refers to the discrete samples f [x,y] representing the continuous image f (x,y). As one of our conventions throughout the class, we will use square brackets [.] when the independent variables x and y are discrete, usually integers.

CLICK

Each element of the 2-d array f [x,y] is called a pixel or pel (from “picture element“). Television engineers generally prefer “pel” – computer engineers prefer “pixels”

Pixels are not squares, rectangles, or circles (even if we draw a picture with squares, rectangles, circles), you should think of them as point samples.  Sometimes, you might hear the term “square pixels,” which is misleading. This does not refer to the shape of a pixel, but an equal horizontal and vertical sampling interval.

To give you some intuition, I show you a head-and-shoulder view sampled with different number of pixels …….

SLIDE 5

Color pictures generally have three color components, which are combined by additive mixing …

CLICK

If you look at a display with a magnifying glass, you see really three displays, one for each color …

CLICK

They correspond roughly to the red, green and blue receptors (“cones”) in the human retina. [false color retina picture, Roorda + Williams, Nature 1990] ….

CLICK

The perception of color results from the difference in R,G,B. For R=G=B, a monochrome image results.

SLIDE 6

So, why do we process images?

CLICK + READ 6 BULLETS

This is not a complete list. Think of image security applications like watermarking, authentication. Or computed imaging: tomography, confocal scanning microscopy. Many students taking this course have their own specific image processing problems that they would like to solve that might or might not fall under one of these headings.

In the next segment, I will show you a number of image processing examples that are related to the material that you will learn in this course.

EE368 Chapter 1 – Introduction – Segment: Image Processing Examples.

Talking head

Welcome back.

In this segment, I want to show you quickly show you some image processing examples. After you complete this course, you should be able to understand and implement any one of these examples. In fact, many of the examples are projects that students did as part of this class in past years.

SLIDE 7

Our first example is image mosaicing. The goal is the take many overlapping images of a scene and stitch them together to a larger pictures. This involves registering the overlapping portions and blending them seamlessly. The top image ….  The bottom image …

SLIDE 8

Another fun image processing example is face morphing ….

SLIDE 9

This example shows a face detection algorithm that was implemented as a student project -- Built into many cameras today, also photo processing software -- Useful to adjust focus, exposure and white balance in cameras -- First stage of face recognition algorithm

SLIDE 10

We did face detection as a competitive term project in this course for a couple of years. Student teams would develop an algorithm to find all the faces in a picture like the one shown here, and we would count how many faces they got right and how many they got wrong.

SLIDE 11

Here is another application for face detection: face blurring for privacy protection in the Google Streetview system. You can see that faces in the picture are detected and blurred to make them unrecognizable. I especially like this example since the algorithm also detects and blurs the face of KFC’s Colonel Sanders.

SLIDE 12

Here is an image processing example that is now part of Stanford history. In 2005, the Stanford robotic car "Stanley" won the DARPA Grand Challenge by completing a 132-mile desert course without a driver. Stanley beat the competitors by a clever image segmentation algorithm that could distinguish  between drivable surface and potential obstacles. Other cars relied on active sensors that could only sense a few tens of feet ahead of the car. Stanley’s segmentation algorithm could see to the horizon which allowed the car to go much faster when the road was straight.

SLIDE 13

Here is another competitive class project that we organized in 2006: Visual Code Marker Recogniton …

SLIDE 14

The year afterwards, we did a competitive class project on painting recognition. Students were given a database of paintings from the Cantor Arts Center at Stanford and developed algorithms to recognize these paintings quickly and accurately.

SLIDE 15

Here is an example of a cameraphone implementation of one of the student projects. …

SLIDE 16

The following year, we did a similar project, but with CD covers rather than paintings. And, again, here is an implementation of a CD cover recognition algorithm on a cameraphone.

SLIDE 17

This implementation plays some music from the album once it recognizes the cover. A yellow quadrangle indicates where the CD cover was detected.

SLIDE 18

After you have seen image processing examples, here is a list of topics we will discuss in this course ….

SLIDE 19

Finally, I’d like to put Image Processing as field into a bigger context. Here is a map I have tried to draw that shows Image Processing relative to other closely related or overlapping fields. Let me point out just a few distinctions:

Image Processing: tends to focus on 2D images, how to transform one image into another, and on image analysis that does not produce semantic interpretations of the image contents.

Computer vision: tends to focus on the 3d scene projected onto one (or several) images. Relies on assumptions about the scene depicted in an image and often strives for some degree of “image understanding.”

Machine vision: tends to focus on applications, mainly in manufacturing/ robotics. Imaging under controlled conditions, image processing, and actuation/control are designed together, as a system.

The focus of this course is, of course, image processing, as the title suggest, even though some of the more advance image analysis techniques we will learn about overlap with computer vision and might also be taught in a Computer Vision course.